Hardware report/Build instructions

Your target audience is a Computer Engineering Technology student from a different college that would like to recreate your project. Guidelines for next semester's CENG 355 Report are available.1

Table of Contents

1.0 Introduction
2.0 Body
2.1 Sensor/Effector purchase
2.2 PCB design and soldering
2.3 Case design and assembly
2.4 Firmware development and use
3.0 Testing and Results
4.0 References

1.0 Introduce the Broadcom development platform and exisiting functionality

This project is based on the Raspberry Pi platform with a custom PCB designed to interface with a VEML7700 ambient light sensor. The system measures real-time light intensity in lux, white light levels, and raw ambient light values. The data is processed via the I²C communication protocol, displayed through a Python script, and can trigger visual indicators when thresholds are exceeded. The objective of this build instruction is to provide a comprehensive guide for recreating the hardware and software environment, ensuring repeatability and accurate system behavior.

2.0 Added functionality

The base Raspberry Pi platform provides general-purpose computing and GPIO/I²C communication. To extend its capabilities, the following functionality was added:

- Integration of the VEML7700 ambient light sensor for accurate lux, white light, and raw ambient readings.

- Implementation of a transistor-driven LED status indicator connected to GPIO17 for external feedback.

- Integration with the Sense HAT onboard RGB LED matrix to provide color-coded status indicators:

  - Green when light levels are sufficient.

  - Red when the sensor is placed in darkness.

- A custom-designed and fabricated PCB to ensure reliable sensor communication and stable power delivery.

- A protective case designed to secure components, expose the sensor window, and allow airflow.

2.1 Sensor/Effector purchase

- Core Platform: Raspberry Pi 4 Model B (2GB RAM or higher recommended).

- Primary Sensor: Vishay VEML7700 ambient light sensor module (supports I²C).

- Supporting Components:

  - NPN transistor (2N4124) for LED switching

  - Status LED (3 mm or 5 mm, green or red)

  - Resistors (220 Ω for LED current limiting, 10 kΩ for pull-up if needed)

  - PCB headers and female jumper wires for I²C connectivity

- Power Supply: Official Raspberry Pi USB-C power adapter (5V, 3A).

- Optional Add-ons: Raspberry Pi Sense HAT (for extended testing of I²C bus functionality).

Suppliers such as Digi-Key, Mouser, or SparkFun can provide these components. The VEML7700 module is also available through Adafruit or Amazon.

2.2 PCB design and soldering

- Software Used: KiCad EDA for PCB layout design.

- Design Steps:

- Create a schematic including the VEML7700 sensor, pull-up resistors for SDA and SCL lines, and a GPIO-connected transistor-driven LED.

- Assign footprints and generate a PCB layout with proper routing for I²C (SDA/SCL) and power lines (3.3V, GND).

- Ensure decoupling capacitors (0.1 µF) are placed near the sensor VCC pin for noise reduction.

- Export Gerber files and submit to a PCB manufacturer (e.g., JLCPCB or OSH Park).

- Assembly:

- Install resistors, transistor, and LED according to the schematic.

- Solder the rest of the connections, and test if are working.

.

2.3 Case design and assembly

- Software Used: Adobe Illustrator for 2D layout and laser-cut design.

- Design Considerations:

- The case must secure the Raspberry Pi and PCB while exposing the VEML7700 sensor window.

- Provide ventilation slots for airflow.

- Include mounting holes for the Raspberry Pi and standoffs for stability.

- Assembly Steps:

- Mount the Raspberry Pi on standoffs inside the enclosure.

- Place the custom PCB in position, ensuring sensor exposure to ambient light.

- Secure the lid and confirm that the status LED is visible externally.

2.4 Firmware development and use

- Base Code: Initial firmware was adapted from the sample code provided in the VEML7700 datasheet.

- Improvements Implemented:

- Added detection of the VEML7700 sensor header to verify proper I²C connection.

- Implemented continuous lux measurement and display.

- Integrated Sense HAT RGB LED feedback logic:

  - If the sensor is placed in darkness, the Sense HAT displays red.

  - Otherwise, the display remains green.

- GPIO17 continues to drive the external LED for hardware-level feedback.

3.0 Testing and Results

Functional Test:

Verify I²C communication using i2cdetect. The VEML7700 should appear at address 0x10.

Run the Python script and confirm live readings of lux, white light, and raw ambient data.

Threshold Test:

Place the sensor under varying light conditions (dark room vs. flashlight beam).

Confirm that the LED indicator toggles correctly when threshold conditions are met.

System Reliability:

Run continuous operation for 24 hours and log results.

Verify stable power delivery and consistent readings without I²C communication drops.

4.0 References

https://cdn-learn.adafruit.com/downloads/pdf/adafruit-veml7700.pdf

Footnotes

- Technology Report Guidelines. OACETT, Revised September 2022. Available at: https://www.oacett.org/getmedia/5ad707d7-f472-4b24-a7fe-f34e270b0c41/2022_TR_Guidelines_-_Updated_Version_-_Sept_2022.pdf ↩